# PLC-based Implementation of Self-improving Q-learning Controller and Validation for Higher-Order Processes

Jakub Musial, Krzysztof Stebel, Jacek Czeczot, Patryk Grelewicz, 
and Bogdan Gabrys, Senior Member IEEE

Abstract—

Index Terms— Q-learning algorithm, reinforcement learning, process control, PLC-based implementation, practical validation.

# INTRODUCTION

T

HIS document

# Motivation and statement of the problem

In this work it is considered how to further improve the performance of the Q-learning controller (designed as Q2d – see [1]) that is dedicated to operate in the industrial closed-loop systems, schematically shown in Fig. 1, as a replacement for the existing PI controller, the control performance of which is not satisfying. Initially, in this system, PI controller with known tunings (gain KPI and integral time TI) controls the Process with the sampling time Ts to stabilize its output Y at the set-point Ysp using the manipulating signal UPI to reject the disturbances d. After replacement, the Q-learning controller takes over the control applying the manipulating signal UQ.

Fig. 1.  Schematic diagram of the considered closed-loop system with the existing PI controller (upper diagram) and its replacement by the Q-learning controller after switching (lower diagram).

The general statement of the problem is the same as for Q2d [1] and it comes down to the following requirements:

- The redesigned Q-learning controller should still provide bumpless replacement of the existing PI controller and this feature should be achievable only based on the knowledge of the existing PI tunings,

- The design of the Q-learning controller should not require any knowledge of the process dynamics,

- The successive improvement in control performance should be possible due to on-line learning during the normal operation of the Q-learning controller without any additional disturbing of the closed loop system,

- The target closed-loop performance should be defined based on the predefined first order target trajectory:

Fe,e=e+1/Tee=0							(1)

with e = Ysp - Y denoting closed-loop error, e its time derivative and Te is the predefined trajectory time constant,

- On-line learning should not introduce even temporary deterioration in the closed loop performance.

These requirements were met for Q2d in the application to limited class of dynamical processes which was shown experimentally in [1] in the application to an example of the 2nd order process. However, the attempts to use Q2d for higher order nonlinear processes show that not all design assumptions are then met. In particular, the following major problems occur:

- During on-line learning, a temporary deterioration in the closed-loop performance can appear in the form of temporary presence of unacceptable steady-state control error e. This phenomenon is shown in Fig. 2. presenting simulation validation of Q2d in the application to a closed-loop system with an example of the 3rd order nonlinear process (A1) representing pneumatic setup described in the Appendix.

- As the process order increases, the ability of accurately achieving the reference trajectory (1) decreases, especially for small values of Te representing desired aggressive closed-loop performance. In such cases, the application of Q2d leads to learning of using aggressive (almost on-off) actions with strongly oscillatory closed-loop behavior.

- The Q2d procedure of generating states and actions is acceptable but does not take into account the limitations existing in the practical control systems. Consequently, Q2d approach is not suitable for PLC-based implementations in the form of the library function block with functionalities similar to e.g. PID function block with limited manipulating variable.

Fig. 2.  Example part of the on-line learning for Q2d showing the temporary occurrence of unacceptable steady-state control error e – simulation results in the application to the 3rd order process (A1).

Addressing these problems is the major motivation for this work where the initial concept of Q2d is significantly improved to its modified Q2dPLC form. For this purpose, the following additional practical design requirements are defined:

- Comparing to Q2d, the learning ability of Q2dPLC should be extended to wider class of dynamical processes,

- The closed-loop performance improvement should continue until the first order target trajectory (1) is reached with the highest accuracy available for a specific controlled process with more complex dynamics. The possible accuracy of reaching this trajectory should be assessed automatically to prevent too aggressive Q2dPLC actions,

- Q2dPLC should be implemented in PLC as a general purpose library function block so the appropriate input-output specification and parametrization should be defined.

The solutions that fulfill these new requirements are presented in this paper and constitute its novelty.

# Modifications of Q2d to Q2PLC

The basic concept of Q2dPLC is identical to Q2d presented in [1] where it is considered as a single agent learning directly from interactions with the environment to keep the environment in the target state by taking the actions selected from the action vector [a] depending on the current value of the state of the environment recognized from the state vector [S]. Then, the current mapping between states and actions is collected in 2-dimentional Q-matrix that indicates the so-far optimal action-state correlation. During the on-line learning and based on the rewarding policy, Q-matrix is updated as:

QS(t),a(t)⃪ QpS(t),a(t)+

+α∙R+γ∙maxa⁡QS(t+Ts),a(t+Ts)--QpS(t),a(t),									(2)

where t is a discrete-time instant, α  [0, 1] is the learning rate, R is the reward assigned to the action a, γ  [0, 1] is the discount factor, maxa selects the optimal action for which the maximum reward is encountered at the next time instant and Ts is the sampling time (for more details readers are referred to [2]).

Same as in Q2d, for the specific environment (here, the closed-loop system shown in Fig. 1), the target state is defined as:

S0=Fe,e=e+1/Te∙e≤prec/Te,			(3)

where prec denotes the acceptable precision of achieving the set-point Ysp in the steady-state but also the target state (here, the closed-loop reference trajectory (1)) in transients. The control law of Q2dPLC is also defined in the incremental form as:

∆UQ=KQ∙Ts∙a,									(4)

where KQ is the adjustable gain of Q2dPLC and Ts is the sampling time taken from PI controller. At each sampling instant, in the exploitation mode the action a is selected based on the Q-matrix as the optimal choice for the current state of the closed-loop system.

## Input-output normalization

In practical implementations in the form of the general library function block it is convenient to ensure that both input and output signals vary within normalized 0-100% range. Thus, for the Q2dPLC it is assumed that both process output Y and its set-point Ysp are normalized to this range outside the function block using scaling functions. Consequently, the control error e and the manipulating variable UQ are also expressed in the same range 0-100%. For the latter, the additional limitation of its range is required in some cases and then UQ  [Umin, Umax] where 0% ≤ Umin < Umax ≤ 100% holds.

## Generating state and action vectors

In Q2d, the state vector [S] was generated assuming hypothetical maximal value of Fe,e. First, assuming a symmetrical distribution of states around the target state S0 and an exponential distribution of the widths of the intervals representing the discretization of the states, the initial state distribution was generated and then the indistinguishable intervals were merged to ensure that the size of the state vector was (2N-1). In the second stage the action vector [a] was generated based on the distribution of states to ensure a symmetrical distribution around action a0 (“do nothing”). This approach is effective but has two important practical limitations. In a general case, it is difficult to assume realistic maximal value of Fe,e and the generated vector of actions may contain actions that are not applicable due to limitations of the manipulating variable UQ  [Umin, Umax].

For Q2dPLC these limitations have been removed by reversing the order of determining both vectors and starting from generating the action vector [a]. It is due to the fact that in practical applications, the aforementioned limitations of the manipulating variable are given and based on Eq. (4), its upper limitation can be used to determine the maximal admissible action amax = (k∙Umax)/(KQ∙Ts) with adjustable k  (0, 1] and the positive part of the action vector can be generated. The actions a0 = 0 (“do nothing” required if the system is in the target state) and a1 = (2∙prec)/Te (necessary for the proper resulting definition of the boundaries of the target state) are always fixed while the subsequent actions are calculated as successive terms of a geometric sequence ai=a1∙qi-1 (for i = 2..N) with a constant common ratio q=amaxa11/N > 1. This procedure is graphically presented in Fig. 3. Finally, the positive actions defined in this way are completed with their negative counterparts to generate the symmetrical complete action vector that consists (2N+1) actions defined as [-aN, -aN-1, …, -a1, a0, a1, … aN-1, aN] with aN = amax.

In the second stage, the vector of states [S] is generated based on the previously generated action vector [a] and the general definition of state:

S=Fe,e=e+1/Te∙e	.						(5)

that represents how far the closed-loop system is from the reference trajectory (1). This continuous state space is discretized and for positive states, the discretized states are defined by the bounded intervals as:

S0:S∈[-s1,s1] – target state, partially negative,

Si: S∈(si,si+1] for i = 1..N-1, positive states,

SN:S>sN, final positive state,

and the successive boundaries are computed based on the previously generated actions as si=ai-ai-12 for i = 1..N, which is also shown in Fig. 3. In the next step the boundaries of the intervals representing negative discretized states are defined as a mirror image of the positive boundaries which forms the complete (2N)-element vector of boundaries representing state discretization as [-sN, -sN-1, …, -s1, s1, … sN-1, sN] which always corresponds to (2N+1) distinguishable states.

Fig. 3.  Representation of generating positive actions and states.

This simple and practically justified procedure proposed for Q2dPLC ensures that for a given closed-loop design parameters prec and Te, the values of N and Umax are the only parameters required for generating the distribution of admissible actions and states. It accurately provides the desired number of (2N+1) accessible actions and states only by adjusting N and regardless of the maximal action amax. This feature is desirable for practical PLC-based implementations when memory usage and computational complexity are the critical limitations. Additionally, the subsequent actions a1 – aN generated in the proposed way increase geometrically, which makes small actions (necessary for precise regulation) and large actions (necessary to quickly regulate the process from one working point to another) available to Q2dPLC.

## Initialization and modifications of learning policy

In Q2d, after bumpless initialization and switching into automatic mode, on-line learning was started immediately for the target reference trajectory defined by Te given as the user-defined parameter. When Te was significantly smaller from the time constant of the closed-loop trajectory of the system controlled by PI controller, the learning was difficult due to the large difference between the existing closed-loop dynamics and the target trajectory defined by Te. Consequently, Q2d-based closed-loop system has a tendency to learn oscillatory behavior and this tendency increases with the increasing complexity of the dynamics of the controlled process. This drawback was partially removed by on-line testing the normalized increment of the norm of Q-matrix [1] but it was difficult to propose a general and reliable rule to interrupt learning to avoid the occurrence of undesirable closed-loop oscillatory behavior.

In this paper for Q2dPLC it is proposed to distinguish between the value of Te representing the current reference trajectory parameter and the Q2dPLC user-defined design parameter Tegoal representing the target reference trajectory that the closed-loop system should reach if it is achievable for a given process with more complex dynamics.

This approach ensures easy bumpless initialization of Q2dPLC because the procedure described in the previous section preserves similarity between the Q2dPLC control law (4) with the state definition (5) and the incremental form of the PI control law ∆UPI=KPI∙Ts∙e+1/Te∙e. Readers should note that when KQ = KPI and Q-matrix is an identity matrix, Q2dPLC ensures the closed-loop performance identical to PI controller with TI = Te with accuracy limited only by states and actions discretization. Thus, by similarity, when Q2dPLC is designed for Te = TI with KQ = KPI and Q-matrix initialized as the identity matrix, the bumpless switching between the existing PI controller and Q2dPLC is ensured in the closed-loop system shown in Fig. 1, without necessity of using any additional recalculations required previously for Q2d. Switching into exploration mode will cause Q2dPLC to achieve the closed-loop trajectory defined by TI as e+1/TIe=0 by the first stage of the on-line learning. Then, further learning also takes place in stages. For conservative PI tuning TI < Tegoal holds which implies that Te should start from Te = TI (for bumpless Q2dPLC initialization) and decrease with the step ΔTe to Te = Tegoal.

Note that even after long learning, it is difficult to ensure accurate closed-loop tracking with the target trajectory Tegoal for processes with more complex dynamics. Thus, the learning process is supervised by introducing the percentage trajectory completion PTC(Te) being the function of the current Te value and indicating the progress of the learning process. Its achievable value varies from ≈ 100% for simple first order process to much lower values (between 20% and 40%) depending on the complexity of the process dynamics and how aggressive the current target trajectory defined by the current Te value is.

The computation of the current value of PTC is based on collecting the information of the level of completion of the target trajectory. This information is coded by the value “1” (“true” for being in the target state S0) and the value “0” (“false” for not being there). These values are set at each sampling instant but only when the closed-loop system is in transients (for steady state with e≤prec the values “1” are always expected and they are not considered to avoid incorrect computation of PTC). The set values “0”/”1” are collected into a vector of a specific length. When the vector is completely filled, the value of PTC(Te) is computed as the arithmetic mean of all vector elements and the vector is emptied for next calculations. The newly computed PTC value is filtered using XXXXXX [], which provides smoothed values of PTC and its time derivative. Each decrement of Te value is applied when PTC(Te) does not change significantly (its time derivative is within acceptable e.g. ±5%) and PTC value drops below the assumed threshold (e.g. when PTC(Te) < 25%). Thus the decrement of Te is interrupted not only when its value reaches Tegoal (which takes place only for processes with simple dynamics and/or not aggressive target trajectory Tegoal). This decrement stops also when PTC reaches its highest value achievable for a specific process with more complex dynamics.

LISTING 1

determine current state St=Fet,et;

if at-Ts==a0		// previous action was “do nothing”

R = 1;			// previous action “do nothing” is rewarded

else

R = 0;			// previous action is not rewarded

if ((R = = 1) or ((previous action at-Ts was randomized) 
and (UQt-TsϵUmin,Umax)))

update Q-matrix for the previous action at-Ts and
the current value of R;

if (St==S0)						// current state is the goal state

adjust at=a0	;		// apply “do nothing” action

else

draw Aϵ0,1;

if (A < eps)											// exploration mode

draw the action at with recommended limitations;

else														// exploitation mode

based on Q-matrix, choose the best action at
				corresponding to the current state St;

compute ∆UQt=KQ∙Ts∙at;

compute UQt=UQt-Ts+∆UQt;

if (UQtϵUmin,Umax				// current manipulating signal

apply UQt;						// within acceptable range

else		// current manipulating signal outside acceptable range

apply UQt=Umin when UQt<Umin
		or UQt=Umax when UQt>Umax;

Apart from aforementioned modifications, there are other novel improvements in on-line learning policy proposed for Q2dPLC presented in LISTING 1 showing the sequence of computations for a single time instant (for clarity, the calculations required for PTC computation are not shown).

The first Q2dPLC modification in learning policy is that learning (Q-matrix update) is possible only for actions that were randomly drawn and they were applied without exceeding the limitations of the manipulating variable Umin,Umax.

The second modification relates to random drawing of actions in exploration mode. The drawing itself is possible only when the system is not in the target state S0. Then, similarly to Q2d, the ε-greedy method [3]-[4] is applied for switching between exploration and exploitation modes but the difference is applied in drawing policy. In Q2d, the range of actions accessible for drawing was limited only by user-defined natural number RD > 0. Consequently, the drawing was allowed only from actions with numbers ±RD around the optimal action indicated by Q-matrix for a current state (see details in [1]). Such an approach meets the requirement to prevent excessive process disturbing by uncontrolled drawing actions significantly different from the optimal action indicated by Q-matrix for a given state. However, Q2d still showed a tendency to learn very aggressive (almost on-off) actions in the application to processes with more complex dynamics. Thus, the drawing policy proposed for Q2d is further modified in Q2dPLC to prevent this problem, which is shown in Fig. 4 for the example value RD = 3.

Fig. 4.  Example part of Q-matrix with the accessible range of randomized actions for Sk state for RD = 3. “X” denotes optimal actions for state Sk and two neighboring states Sk-1 on the left and Sk+1 on the right. Red arrows show the ranges of actions accessible for drawing for each state while green arrow shows the effective range of actions accessible for drawing for the state Sk after applying the modified drawing policy proposed for Q2dPLC (see details in the text).

Now, for the system being in the state Sk with the currently assigned optimal action aj, the range of actions accessible for drawing is determined not only based on the action range for Sk and a given RD (here, aj-3..aj+3), how it would be for Q2d, but also on the action ranges corresponding to two neighboring states: on the left Sk-1 with assigned optimal action aj-1 (action range aj-4..aj+2) and on the right Sk+1 with assigned optimal action aj+2 (action range aj-1..aj+5) – see red arrows representing these ranges of actions in Fig. 4. Then, the final range of actions accessible for drawing for the system being in the state Sk is determined as the common part of these three ranges and is marked with green arrow (here, aj-1..aj+2). This range is smaller so effectively fewer actions can be drawn comparing to Q2d approach. However, this limitation proposed for Q2dPLC ensures smoother changes of actions during on-line learning and thus prevents uncontrolled learning of aggressive closed-loop behavior.

Finally, no matter how the current action was selected, the final current value of the manipulating variable is always kept within the acceptable range Umin,Umax.

Summarizing, these novel modifications proposed for Q2dPLC solve the practical problems that occur with previous Q2d approach:

- Q2dPLC initialization for bumpless switching from PI control is as simple as for Q2d (assuming Q-matrix as identity one) but does not require recalculations that were necessary for the latter,

- Consecutive decreasing Te ensures that Q2dPLC does not tend to learn aggressive actions that takes place for Q2d for the processes with more complex dynamics and when Tegoal is too aggressive,

- Comparing to Q2d, modification of learning policy provides additional prevention from learning aggressive actions but it also prevents from incidental appearing of the steady state control error at the initial stage of learning when Q2dPLC is implemented for the processes with more complex dynamics.

The only price for these improvements is that for each update of Te, the generation of [a] and [s] vectors must be repeated for the same N, amax and prec to adapt to the new target trajectory. However, the generation procedure has low computational complexity and it can be easily implemented in PLC.

# PLC-based implementation and validation

- Appendix

Practical validation of the proposed approach was based on the laboratory pneumatic setup shown in Fig. X1 and representing the third order dynamical process that consists of three serially connected tanks of the respective volumes V1 = 5 [L], V2 = 2 [L] and V3 = 0.75 [L]. The relative pressure in each tank is respectively denoted as p1, p2, p3 [bar]. The system is supplied with the air of the relative pressure ps [bar] and the air flows between the tanks through the constant pneumatic resistances Rpa, Rpb, Rpc [1/(m∙s)]. The air flows out from the last tank through the pneumatic resistance Rpout [1/(m∙s)]. The input (manipulating) variable is the supplying pressure U = ps and the output is defined as Y = p3. The pneumatic process can be also disturbed by applying the load disturbance emulated by adding Δps directly to the manipulating variable.

The supplying relative pressure ps is adjusted by the proportional valve MPPES-3-1 from Festo® within the range 0 - 5 [bar]. All the pressures ps, p1, p2, p3 are measured on-line by the SDE1 pressure sensors. All process signals are connected to analog I/O modules in ET-200 module connected to PLC- SIEMENS SIMATIC® S7-1500, which is the hardware platform for implementation of Q2dPLC.

Fig. X1.  Laboratory pneumatic setup and its simplified scheme with hardware configuration of the experimental setup.

For simulation experiments, the dynamics of the considered pneumatic setup can be described by the following nonlinear state space model:

p1=0.994∙U-p19.38p2=0.972∙p1-p22.34p3=0.081∙p22+0.4∙p2-p31.55,					(A1)

with parameters identified from consecutive series of step responses withing the whole operating range using MATLAB library function fminsearch().

- References

J. Musial, K. Stebel, J. Czeczot, P. Nowak, and B. Gabrys, “Application of self-improving Q-learning controller for a class of dynamical processes: Implementation aspects,” Applied Soft Computing, 152, 2024, 111250.

B. Jang, M. Kim, G. Harerimana, and J.W. Kim, “Q-learning algorithms: a comprehensive classification and applications,” IEEE Access, 7, 2019, 133653-133667.

S. Syafiie, F. Tadeo, and E. Martinez, “Softmax and "ε-greedy” policies applied to process control,” IFAC Proceedings, 37, 2014, 729–734.

H.K. Lam, Q. Shi, B. Xiao, and S.H. Tsai, “Adaptive PID Controller Based on Q-learning Algorithm,” CAAI Transactions on Intelligence Technology, 3(4), 2018, 235–244.

Manuscript received XXXX; revised Month xx, xxxx; accepted Month x, xxxx. This work was supported in part by the grant from SUT - subsidy for maintaining and developing the research potential in 2025 (J. Musial, K. Stebel, J. Czeczot, P. Grelewicz) (Corresponding author: Jacek Czeczot)

J. Musial. K. Stebel, J. Czeczot and P. Grelewicz are with Silesian University of Technology, Faculty of Automatic Control, Electronics and Computer Science, Department of Automatic Control and Robotics, Gliwice 44100, Poland. (email: Jakub.Musial@polsl.pl; Krzysztof.Stebel@polsl.pl; Jacek.Czeczot@polsl.pl; Patryk.Grelewicz@polsl.pl).

B. Gabrys is with University of Technology Sydney, Faculty of Engineering and IT, Advanced Analytics Institute, New South Wales 2007, Australia. (email: Bogdan.Gabrys@uts.edu.au).