https://www.astrj.com/  storna czasopisma

Zalecenia:

The text body should not be formatted, for example, splitting in columns, adding headers and footers is forbidden.
Nomenclature or abbreviations list is not needed – all explanations should be placed in the text.
Recommendation section is not allowed.

The Volume of the Article
The volume of the article (including tables and illustrations) should not exceed 20 pages of A4 format (included in the fee).
Placing longer publication requires approval by the Editors, and pay an additional fee.

Model-Free Q-Learning Controller with Projection Function and Dead Time Compensation for Industrial Process Control

Abstrakt:

Introduction:

Statement of the problem

Q-learning is a model-free reinforcement learning algorithm that learns optimal control policy which bring system to goal-state, through interaction with its environment. Algorithm observe current state s, select an action a, transition to new state s’ and receive scalar reward for applied action. The actions are selected from set of predetermined action table based on the information collected in Q-matrix, whose elements indicate what action will bring state of the process closest to goal state. In standard approach in the initial stage of work Q-learning doesn’t know anything about its environment and learn it by randomly applying actions and rewarding those which bring system to goal state. After that, actions that lead to non-goal near goal-state will also be rewarded by part of reward value. Q-matrix is updated according to following formula:

Qst,at⃪Qpst,at+αR+γ*maxa⁡Qst+1,at+1-Qpst,at,			(1)

Here, t denotes a discrete-time instant, s represents the system state belonging to a predefined set of states s ∈ S, and a denotes an action selected from a predefined set of actions a ∈ A. The action is chosen either as the optimal one during exploitation phase or drawn during exploration phase and is applied in the current state. The term Q(st, a) denotes Q-value, the corresponding element of the Q-matrix, which represent weights for applying an action a when system is in state s. In Eq. 1 Qp(st, a) denotes the Q-value prior to the update, while Q(st+1, at+1) corresponds to the Q-value of the subsequent state-action pair after executing action at. The parameter α ∈ [0, 1] is the learning rate, R is the reward associated with state st, and γ ∈ [0, 1] denotes the discount factor that determines thew relative importance of immediate versus future reward. The operator maxa selects the action that yields the maximum expected reward at the next time step. For further details on Q-learning algorithm, their theoretical foundation and a comprehensive review of their applications, the reader is referred to (B. Jang, 2019)

Controller design:

In control engineering application, one of the most promising approaches is the direct implementation of single-agent Q-learning algorithm as controller within a conventional single-loop closed-loop control system. In the framework, the Q-learning controller (agent) learns directly through interactions with controlled Process (environment), based on current state of closed-loop system, which is defined using the control error and the dynamics of its decay.

In most industrial closed loop systems, control is usually performed by a PI controller that is often poorly tuned. The objective of this work is to propose self-improving Q-learning-based controller capable of replacing the existing PI controller. Their objective is stabilization, which aims to maintain the process output Y at its desired setpoint YSP (tracking) by adjusting the manipulated variable U in the presence of varying load disturbance d.

The Pi controller is pre-tuned, with known tuning parameters and provides acceptable but suboptimal control performance, which may significantly deviate from the desired closed loop behavior. Since the process model is unknown, the Q-learning controller must initially preserve the same or varying similar control performance immediately after replacing the PI controller. While controlling the process, during the exploration phase, the controller is expected to progressively improve its performance and achieve the desired target behavior by learning directly from dynamic process, which is key characteristic of Q-learning approach.

This article assumes that the process is characterized by unknown, time-invariant first-order dynamics with time delay, developing the concept presented in my previous work [?], in which the lack of time delay compensation was a serious problem due to its widespread occurrence in industrial processes.

The only information about the process available to the algorithm is the PI controller tuning, which should be used to initialize the method appropriately to ensure smooth switching between controllers. Additionally, it is assumed that the algorithm has a predefined and discretized state space appropriate for the process and a set of available actions.

Most industrial closed loops are designed to stabilize the process with rare changes in the setpoint YSP. This leads to the assumption that the main task of the control algorithm is to suppress disturbances occurring in the process in the form of disturbances of roughly known amplitude. Such assumptions guarantee the occurrence of disturbances during which the controller can learn online without the need to artificially disturb the process. For this reason, in this work, only load disturbances with randomized amplitude and duration of action on the closed-loop system were used to train the algorithm.

Continuing the assumptions from previous works, the goal of the control is for the system to follow a given first-order trajectory with error decay defined as Fe,e=e+(1/Te)e, with e denoting control error and Te signifying the time constant of the target trajectory. This description of the control objective ensures ease of design of the target control trajectory, its robustness, and performance for a wide range of processes.

Dead time challenge:

Many industrial processes exhibit significant dead time (also called transport delay or time delay) between control action and measured output:

Yt=Gs∙e-T0s∙U(t)

Where G(s) is transfer function without the delay and T0 > 0 is dead time. Physical sources of dead time include material transport through pipes or conveyors, distant-velocity lags in continuous process, measurement sensor delay, actuator respond time, communication latency in distributed control systems etc. Dead time poses fundamental challenge for Q-learning based controller because update rule (1) assumes that the consequence of action a(t) is observable in the next state s(t + Ts), When dead time spans through multiple sampling intervals (T0 >> Ts) the control action ΔUQ(t) only affects the measured output at time t + T0. This creates the credit assignment problem – when the algorithm assign reward to actions that effect will we visible in the future. Without proper credit assignment, Q-learning may update the wrong state-action pairs which leads to poor or failed learning. In next sections I will address this through delayed credit assignment using FIFO buffers that defer Q-value updates until action consequences become observable.

Controller design:

We consider a single-input single-output (SISO) dynamic process with control signal U(t), process output Y(t), desired setpoint YSP(t) and control error e(t)=YSP(t) – Y(t). The objective of control is to drive the error near zero to predefined (-precision, precision) zone around it. We defined the desired closed-loop behavior through the first order trajectory:

Fe,e=e+1/Tee=0 (2)

Where Te > 0 is desired closed-loop time constant and ė is the error derivative. The analytical solution to (2) is:

et=e0exp(-tTe )   (3)

Which represents the exponential decay of error with time constant Te. Smaller values correspond to faster response but require more aggressive control actions.

This trajectory formulation provides an intuitive link to existing PI control law which can be described as:

ΔUPIt=KPITset+1TIet   (4)

Where KPI is the proportional gain, TI is the integral time and Ts is sampling time. Based on that the control output for designed controller is:

ΔUQt=KQTs∙s=KQTset+1Teet   (5)

And after assuming KQ = KPI the similarities between formula (4) and (5) become visible.

In previous work the Q2d idea was presented which reduced the number of dimensions from 3 to 2 providing faster learning rate and easier industrial implementation. This approach creates states by merging error and its derivative into single state variable based on target trajectory:

s=e+1Tee   (6)

This merged state has clear physical interpretation: s measure the deviation from desired first-order trajectory. When s=0, the error evolves according to e= -(1/Te)e, meaning the system follows the target trajectory exactly. Positive values s>0 meaning the error is decreasing slower than desired and when s<0, the opposite. State table S accommodates both negative and positive values and represents the discrete state space as:

S = {s1, s2, … , sN}  (7)

Where N is total number of states, typically even number to include symmetrical states on both sides of 0.

State and action generation:

To provide efficient scaling and desired precision properties in this paper we propose geometric distribution of states and actions space. The creation of state and action spaces begins with the creation of an action vector, based on which the state space will be created. We start by determining the smallest and largest action values for a given process. Then, using a geometric sequence, actions between these two extremes are generated. This creates an action vector whose values are the means of the state table intervals. This table stores the value limits for each state. This division of space guarantees very simple subsequent initialization of the Q matrix, ensuring bumpless switching between PI and Q2d. Smallest action is described as:

amin=2 ∙precisionTe (8)

Where precision is smallest acceptable value of error in module. Factor 2 ensures that when states are placed at midpoints, the smallest state becomes precision/Te, which guarantees ±error tolerance. The upper control value can be calculated as:

Uscaled= Umax [%]KQTs (9)

Where Umax is usually 100% and KQ and Ts account for controller gain and sampling time. Next the geometric ratio can be defined:

q=Uscaledamin1/(N-1) (10)

Where N is number of desired positive actions. Based on previous steps the positive side of action table can be calculated:

a1=0 (11)

a2= 2 ∙ precisionTe

ai=a2∙qi-1 for i=3, 4, …, N

This creates dense action space near 0 (precise control) and coarse spacing at large values (reducing number of actions).

States are placed at midpoint between consecutive actions:

si=ai+1+ai2  for i=1, 2, …, N-1 (12)

The result is N-1 states where the smallest positive state boarder is:

s1=a2+a12=a22=precisionTe  (13)

Then to handle negative errors and control signal reduction both spaces can be extended symmetrically:

S=-sN-1, -sN-2, …,-s1, s1, …, sN-1 (14)

A=[-aN, -aN-1, …, -a2, a1=0, a2, …,aN]

Resulting in 2N states, 2N+1 actions, goal state index = N+1 and goal action index = N+1.

This discretization method guaranty Te-invariant error tolerance, in steady state: e=0, so state value becomes:

ess=1Teess (15)

The goal state spans s ∈(-s1, s1] where s1=precisonTe. Substituting:

-precisionTe<essTe<precisonTe = -precison<ess<precison (16)

The steady state error tolerance remains ±precision

Q matrix initialization:

The Q-matrix is initialized as an identity matrix:

Qij= 1 if i=j0 otherwise (17)

Where i indexed states and j indexed actions. This ensures that Q-controller in each state initially selects the action corresponding to its own value, mimicking the Pi controller. The diagonal initialization provides stable starting behavior while allowing the learning process to discover improved policies.

Projection function

The primary function of projection function is to enable bumpless switching from existing PI controller to the Q2d controller even when the desired trajectory time constant Te is different from the baseline integral time TI. Without it to, bumpless initialization required Te = TI which restrict controller learning capability. This approach was described in detail [?]. In summary the projection function project ΔUQ actions based on Te to ΔUPI actions based on TI to guarantee bumpless switching.

ΔUPI=KPITse+1TIe (22)

ΔUQ=KQTse+1Tee   (23)

When KPI = KQ, the difference is:

KQTse+1Tee- KPITse+1TIe=e 1Te-1TI (24)

So modifying control law by this difference, bumpless switching is possible:

ΔUQ=KQTse+1Tee- e 1Te-1TI   (25)

The complete Q2d control law with projection becomes:

Ut=Ut-Ts+KQ∙as∙Ts-e(t)∙1Te-1TI (22)

Where a(s) is action selected for current state s(t). The control signal is saturated:

Ut=satUmin, UmaxUt

With typical bounds Umin=0% and Umax=100% for industrial application.

For improve numerical precision, the projection is implemented using the continues error value. To ensure stability near the setpoint this function must be disabled in goal state and its surrounding states. This is because controller work on quantized states where goal state has set precision. Using continues error value in projection function to generate ΔU=0 in goal state will be only possible where e, de = 0 which miss the point of precision around SP. In states next to goal state often projection function change sign of control increment and influence learning patterns.

Dead time compensation via delayed credit assignment

Dead time parameters

Industrial processes often exhibit dead times that may be imperfectly known or difficult to measure precisely. To enable systematic investigation of dead time compensation strategies and robustness to estimate errors, two independent dead time parameters are introduced:

- T0 (Plant dead time): The physical dead time in the actual process, representing reality. The control signal U(t) affects the process at time t+T0. This parameter is inherited to the process and cannot be changed by the controller.

- T0,controller (Controller compensation dead time): The dead time used by controller for credits assignment. This represents the controller’s assumption or estimate of process dead time, which may differ from the true value T0.

This decoupling enables investigation of several practically relevant scenarios:

- Matched compensation (T0, controller = T0): The controller has perfect knowledge of dead time and compensates optimally.

- No compensation (T0, controller = 0): The controller ignores dead time entirely, relying on implicit learning to handle the delay.

- Undercompensation (T0, controller < T0): The controller underestimates the dead time, crediting actions too early.

The ability to independently vary these parameters provides insight into the method’s robustness – a critical consideration for industrial deployment where exact dead time knowledge is not always available.

Dealy credit assignment algorithm:

The core challenge in Q-learning with dead time is ensuring that Q-value update credits the correct state-action pairs. When dead time spans multiple sampling intervals, the effect of action a(t) taken at time t becomes observable only at time t + T0.

Standard Q-learning assumes single step transition

Qst,at⃪Qpst,at+αR+γ*maxa⁡Qst+1,at+1-Qpst,at (18)

Where st is the state at time t, at id the action taken, and st+1 is the state observed at t+T0, not at the next sampling instant t + Ts. This implicitly assumes that st+1 reflects the consequence of at. When dead time exists this assumption breaks. Applying (18) would incorrectly associate st+1 with at, leading to poor or failed learning process.

To resolve the credit assignment problem FIFO buffers were introduced that store state-action pairs and delay Q-value updates until the results of given action become observable. The buffer size is determined by the controller compensation strategy:

Nbuffer=T0, controllerTs+1 (19)

The addition sample (+1) account for the discrete time update sequence: the plant output Y(k) at iteration k reflect the control signal from iteration k – Nbuffer, requiring one additional buffer element for correct temporal alignment.

To further explain delayed credit assignment, consider timeline of events:

- Time t: Controller observe state s(t), select action a(t), computes control U(t)

- Time t: Control U(t) enters the plant dead time buffer of length T0 / Ts

- Time t: State-action pair enters controllers’ buffer of length T0, controller / Ts

- Time t + T0: Plant output Y(t + T0) reflects the effect of U(t)

- Time t + T0: State s(t+T0) computed from Y(t+T0) reflects consequences of a(t)

- Time t + T0, controller: Controller buffer pops (s(t), a(t)) for the Q-value update

- Time t + T0, controller = T0: The update at time t+T0 pairs a(t) with s(t+T0)

This synchronization ensures correct credit assignment: the action a(t) is updated based on the state s(t+T0) that reflects its consequence.

Reward strategy:

The choice of reward function significantly influencing Q-learning convergence and the learning process. Reward strategy that provides positive reward only at the goal state was employed:

Rs,a= 1 if s=sgoal and a=agoal0 otherwise (20)

Where sgoal is the discrete state closest to s=0 and agoal is the zero control increment action.

This reward design ensures that:

- Direct reward at goal: The Q(sgoal, agoal) will always be the highest value in whole Q-table, non-other state-action pair can achieve the max value of goal pair.

- Bootstrapped learning: All other Q-values are learned through the bootstrap term γ maxa Q(st+1, at+1) in (18). States that frequently transition to states with high Q-values will themselves acquire high Q-values, propagating the goal value backwards through the state space.

- Gola state preference: Because only the goal state provide direct reward, the controller leans to drive the system toward s=0 (the target trajectory) rather than lingering in off-trajectory states.

- Zero action enforcement: By rewarding only when a=agoal, we prevent the controller from selecting non-zero action in the goal state, which would unnecessarily disturb the steady-state operation.

- Convergence properties: Uder presented reward policy, the Q-value in goal state approaches:

Qsgoal, agoal=11-γ  (21)

As learning progress and the goal state is visited repeatably. For γ=0.99 the theoretical limit is Qsgoal, agoal=100. States further from goal acquire lower Q-values based on the expected cumulative discounted reward for following the optimal trajectory towards the goal. This creates the gradient in the Q-matrix that guides the controller toward the target trajectory.

As in previous works the exploration/exploitation problem is addressed using ε-greedy method [?]-[?]. In principle, achieving the highest possible learning speed during the exploration phase would require an unrestricted range of randomized actions. However, this is impractical in industrial environments, where learning must occur under normal operating conditions. To address this limitation, the action selection range is constrained by parameter RD [?]-[?]. This parameter defines the allowable deviation range around the action with highest Q-value in Q-matrix. A smaller RD reduces the intentional disturbances introduced into controller process during online training but comes with the cost of slower learning rate. In addition, the action randomness was reduced further by enforcing the same sign action as highest Q-value one. This preventing changing ΔU sign near goal state.

Validation and results:

Validation of the proposed Q2d controller with projection function and dead time compensation was done through simulation studies on two benchmarks models representing common industrial dynamics. All experiments were conducted in MATLAB/Simulink environment with sampling time Ts = 0.1s.

Model – first order inertia:

G1s=kTs+1e-T0s=15s+1e-T0s (26)

This represents simple thermal processes, flow control or level processes where dominant dynamics are characterized by a single time constant.

Dead time scenario

Three dead-time values scenarios were investigated:

- T0 = 0s: baseline case without deadtime

- T0 = 1s: Moderate deadtime

- T0 = 2s: Significant deadtime near half of time constant

For each three compensation strategies were performed:

- No compensation: T0, controller = 0

- Udercompensation:  T0, controller = T0/2   +/- 20%

- Matched compensation: T0, controller = T0

- Overcompensation

PI controller tuning in all experiments were set to Siemens PLC default tunings:

- KPI = 1

- TI = 20

Q2d parameters were set as follows:

| Parameter | Symbol | Value |
| --- | --- | --- |
| Controller gain | KQ | 1 (=KPI) |
| Goal trajectory time constant | Te | 5s |
| PI Integral Time | TI | 20 s |
| Learning rate | α | 0.1 |
| Discount factor | γ | 0.99 |
| Exploration rate | ϵ | 0.3 |
| Random deviation | RD | 3 |
| Precision | precision | 0.1% |
| Number of states | N | 50 |
| Sampling time | Ts | 0.1 |
| Control limits | [Umin, Umax] | [0%, 100%] |
| Set point | SP | 50% |

Every test scenario was performed on load disturbance randomly drawn from normal distribution with σd=0.53 (three sigma rule) and μ=0. Time of disturbance was also randomize using normal distribution σN=150 and μ=3000. This means that not every disturbance will be regulated before new will be generated. Using normal distribution is way to mimic most common disturbance appearance in real industrial systems.

After every 100 epoch of learning (100 generated disturbances) the verification experiment was performed when ϵ=0 to allow comparation without random action exploration. At the beginning of each experiment SP was changed to 20% and controller was put in manual mode to set process output Y to 20%. Three phases of experiment were performed:

- SP change to 50%

- Generating disturbance d=0.3

- Stop generating disturbance d=0

This guaranty that every experiment can be compared to each other and following metrics could be computed:

- Integral Absolute Error (IAE): 0Tetdt

- Maximum Overshoot: maxtYt-YSP

- Settling time: Time to reach ±2% of final value

Maximum control Increment: maxt|ΔUQt|

# Bibliografia

B. Jang, M. K. (2019). Q-learning algorithms: a comprehensive classification and applications. IEEE Access 7, pp. 133653-133667.