Implementation aspects of Q-learning controller for a class of dynamical processes

Jakub Musiał 
Silesian University of Technology, Faculty of Automatic Control, Electronics and Computer Science, Department of Automatic Control and Robotics,

44-100 Gliwice, Poland. Jakub.Musial@polsl.pl

Krzysztof Stebel
Silesian University of Technology, Faculty of Automatic Control, Electronics and Computer Science, Department of Automatic Control and Robotics,

44-100 Gliwice, Poland. Krzysztof.Stebel@polsl.pl

Jacek Czeczot
Silesian University of Technology, Faculty of Automatic Control, Electronics and Computer Science, Department of Automatic Control and Robotics,

44-100 Gliwice, Poland. Jacek.Czeczot@polsl.pl

Abstract—This paper presents a new approach to the general-purpose self-improving controller based on Q-learning control strategies. The previous approach was based on a three-dimensional Q-matrix, significantly slowing down the learning process and limiting the ability of practical implementation in industrial practice. The proposed new algorithm solves this problem by reducing the size of the matrix and the number of tuning parameters of the method without sacrificing its accuracy and learning capabilities. The algorithm is validated by simulation on the two second-order dynamics models and the results show significant improvement compared to the previous version of the developed method.

Keywords—process control, Q-learning algorithm, reinforcement learning, online learning, intelligent control.

# Introduction

Q-learning algorithm is widely investigated and applied by researchers to different applications. It was proposed as an effective reinforcement learning (RL) method by [1-2] and currently, only according to the Web of Science database, 753 scientific articles related to this topic have been published within 2019-2022. Among these articles, 93 are directly related to the control of various processes, which shows the potential of this methodology not only for applications to discrete systems but also to process automation. The algorithm itself was considered from a theoretical point of view [3-6] in terms of stability, learning dynamics, etc. There are also many applications in the literature for discrete-time systems [7-8]. In [9-10] the similarity between the Q-learning algorithm and the MPC (model predictive control) algorithm can be shown in application to dynamical discrete-time systems. Both algorithms optimize the same quality index and they use predefined closed loop trajectory, both for linear and nonlinear systems. Based on this similarity, it is proposed how to compute the decoupled Q matrix for individual inputs for multi-input systems. However, readers should note that despite the similarities between MPC and Q-learning, the cases in which they can be used in practice are extremely different. MPC is preferred when process model of relatively high accuracy is known so there is no need to apply any on-line learning because dedicated MPC design ensures that all the knowledge about the process in directly included in the final control law. Q-learning is dedicated to cases when model is unknown and on-line learning is the only possibility to improve initial (usually poor) control performance. For continuous processes, the literature is very rich in application examples that concern e.g. the control of domestic hot water systems [11] where the Q-algorithm is directly used as a controller. The work [12] addresses important issues from an application point of view regarding the influence of measurement noise on the learning process. Another important practical issue is the initialization of the Q-matrix. According to [13] convergence of the Q-matrix and thus the learning rate depends on its initial value. In that work, it is determined by learning based on simulation, which is an important practical limitation due to the frequent lack of a detailed model of the process being controlled. In [14] authors also deal with the problem of initialization of the Q matrix and they propose Flower Pollination Algorithm for this purpose. This approach allows for taking into account forbidden zones which are important for robot control. Another work dealing with implementation aspects is [15] in which an algorithm for industrial furnace control is proposed. Authors show that providing the correct Q-matrix initialization for systems with unknown dynamics was not developed so far, which creates an important research gap. At the same time, initialization based on knowledge of the process operator is proposed. Namely, the decision tree of actions and results is reconstructed from historical process data and then, the complex algorithm allows for initializing the Q-matrix. However, from a practical viewpoint, that approach has many limitations and is very complex.

Contrary to other reports, in this paper, the authors concentrate on the practical implementation aspects of a Q-learning-based controller. In previous works [16-17] it was shown that the classical Q-learning controller is analogous to an integral controller (I) with variable gain that operates on the error of the process which is the instantaneous value of e.g. temperature, displacement, voltage or any other quantity that characterizes well the static properties of the process. This definition of the state is popular in many works e.g. [14-15]. From the control theory point of view, it is very important to know not only the value of the control error but also its derivative. The introduction of the control error derivative to the Q-learning algorithm opened new possibilities in terms of defining a trajectory along which the control system should move towards the steady-state. In many industrial applications, in addition to proper initialization, the ability to specify such a trajectory is a key implementation requirement.

In the practice, in the majority of cases, the process is usually controlled by the conventional PI controller that usually is poorly tuned. Thus, the concept is to substitute this controller with a self-improving Q-learning controller that at the initial stage provides very similar performance and can provide successive improvement due to learning ability. The existing PI controller can have different forms [18-19] but the information contained in its tuning parameters is very important for the proper initialization of the Q-learning controller without any knowledge about process dynamics. The introduction of the derivative of the control error to the algorithm, however, entailed the disadvantage of changing the Q-matrix from 2-dimensional to 3-dimensional, which significantly increases the number of admissible states and significantly reduces the learning rate, which should be considered a disadvantage. What is new compared to the literature and previous works of the authors is the preservation of the advantages of the proposed approach while partially reducing implementation obstacles. These novelties include the introduction of a new definition of the system state making the Q-matrix 2-dimensional again. An algorithm for reducing the number of states simultaneously providing expected control accuracy is introduced. The possibility of proper initialization of the Q matrix based on the tuning settings of the PI controller is introduced. Due to proper initialization and reduction of the Q-matrix size, a significant acceleration of the learning process is obtained.

# Statement of the problem

As it was said in the introduction, this paper extends previous work [17] in which preliminary modifications for the efficient application of Q-learning controller to process automation were reported. In that work, 3-dimentional Q-matrix was proposed (in this work called Q3d) to include information about both control error and its time derivative. Then, for such a Q-matrix, an initialization based on the behavior of the currently running PI controller in the process was proposed. It was also shown that this approach provides a continuous improvement in control performance compared to the existing PI controller. An important limitation was that the proposed solution is difficult to implement practically in industry e.g. in a Programmable Logic Controller (PLC). The 3-dimensional Q matrix makes it possible to distinguish the states of the process with high accuracy, but the large amount of data required significantly extends the learning process. At the same time, it requires the adjustment of many parameters that are case-dependent and in practice, their values are highly uncertain.

The new approach proposed in this paper (called here Q2d) successfully solves most of the aforementioned difficulties. The control goal remains unchanged and it consists in stabilizing process output Y at the set-point Ysp by manipulating the control signal U. It is still assumed that Q2d should be able to bumplessly replace the existing PI controller but with acceptable and limited accuracy. Finally, very similar generality and improvement in the control performance by online learning should be obtained compared to Q3d.

# Q2d approach to Q-learning controller

The concept of the suggested Q2d approach is still based on the definition of the closed-loop first-order target trajectory defined as a target state in the same way as for Q3d [17]:

Fe,e=e+(1/Te)e=0,                  	(1)

With Te denoting the time constant of the target. For Q3d, separate state spaces were defined both for the control error and for its derivative, which resulted in a 3-dimensional Q matrix. For Q2d, based on (1), it is decided to reduce the dimension of the Q-matrix by merging these values into a single state. This way of defining the state significantly simplifies the subsequent initialization of the Q-matrix based on the required similarity with the existing PI controller. If the output of the PI controller is defined by the recursive formula:

UPI= UPI-1+KPI* Ts  e+(1/TI)e,	(2)

where KPI, TI and Ts respectively denote PI gain, integral time and sampling time, then the control output for the designed Q2d can be expressed by similarity as:

UQ= UQ-1+KQ* Ts*s,	          (3)

where KQ denotes the adjustable gain of the Q-learning controller and s denotes the predefined state defined as:

s= e+(1/TI)e.		(4)

Then, after assuming KQ = KPI, the similarity between both formulas is clear. The creation of state space is crucial and it can be implemented in various ways depending on the implementation capabilities and the expected accuracy of the control after the learning stage. The rule of thumb is that increasing the number of states allows for more accurate distinction between system states and consequently more precise control, however at the high price of learning time and memory usage (larger Q-matrix size). In the Q3d approach [17], the state space was created to ensure similarity to the conventional PI controller, which facilitates the initialization of the Q-matrix. Then, the size of the resulting 3-dimentional Q-matrix with separate arrays for actions, error state and error derivative state. The parametrization was done using four parameters with uneven state distribution getting denser for small values of the control error to provide more accurate control as the system reaches the goal state.

In this paper, a new Q2d approach is proposed to reduce the number of parameters required for efficient parameterization only to two, without significant loss of accuracy. In addition, the state creating process is significantly simplified. After general defining the state value s as given by (4), this value must be discretized to derive the boundaries of each interval in the state space. It is based on determining two vectors of discretized values of the control error and its derivative. This procedure can be carried out in different ways depending on the requirements. In this paper, this discretization is based on the assumption of an exponential control error decay trajectory, which ensures that the state discrimination accuracy increases as the control error value decreases. This trajectory is parameterized by the parameter τ defining the decay ratio of the trajectory and, consequently, the number of distinguished intervals. Discretization is made as follows:

i = 0, t = 0

REPEAT

INC(i), t = t+Ts

ei=emaxe-t/τ

ei=(emax/Ts)e-t/τ

UNTIL ei+(1/TI)ei<precision

imax = i,

and the values of emax and precision should be chosen according to the technological requirements and expected precision of control. Parameter emax  defines the maximum expected value of the control error that can occur in the control system. Thus, it allows for initialization for setpoint changes within the range ±emax. Parameter precision directly defines the precision of reaching the setpoint.

After completing the discretization procedure, two vectors of ei and ei are computed and both vectors are the same size i = 1..imax. Then, boundaries of state intervals sk are also computed:

k = 0,

FOR i = 1 to imax

FOR j = 1 to imax

INC(k)

sk= ej+(1/TI)ei

kmax = k.

Potentially, such computed kmax boundaries can define state intervals that are very small and whose boundaries are very close so, in practice, they can be indistinguishable. This fact can significantly limit the learning abilities of the Q2d learning controller. Thus, state intervals that are close with predefined accuracy (e.g. one can use previously defined precision parameter) are merged, which results in N < kmax distinguishable state intervals. Finally, the vector of boundaries of these intervals should be extended symmetrically as a mirror image to include negative values at the beginning of the vector. Then, the goal state is always located in the middle of the vector and, for calculating the actions of the Q2d controller, the mean states sm must be calculated as:

sim=(si+1+ si)/2       for i = 1..(2N-1).	(5)

For such a state space, the 2-dimentional Q-matrix size is (2N-1) x (2N-1). This approach allows for a very simple initialization of this matrix to ensure initial performance very close to the existing PI controller. This initialization simply consists in creating Q-matrix as the identity one. Thus, based on (3), for each state, the increment of the control signal ΔUQ = sm*Ts*KQ, which is identical to the PI controller with discretization accuracy. The aforementioned state definition significantly facilitates Q-matrix initialization to ensure bumpless switching between the operating PI controller and the proposed Q2d controller. However, it does not ensure that the goal state represents desired reference trajectory given by (1). Thus, while the normal operation of the Q2d controller, the current value of the system state must be computed as:

s= e+(1/Te)e,		(6)

and the control signal given by (3) must be recalculated as:

UQ= UQ-1+KQ* Ts*s-e1/Te-1/TI      (7)

Similar to the previous Q3d approach, the exploration/exploitation problem is solved by using the ε-greedy method [20][21]. For possibly fastest learning, in the exploration stage, the range of randomized actions should be unlimited, which is unacceptable in industrial practice when learning must be carried out based on a normally operating process. Thus, the range of actions drawing is limited by the parameter RD (for details, see [17]), which defines the limits of the range of draws around the optimal action represented by the highest value in the Q-matrix. The smaller RD is, the smaller disturbances appear in the controlled process during learning in the normal operating, but at the price of extending the learning time. In addition, in the situation when desired reference trajectory is very different from the trajectory ensured by the existing PI controller, for too small value of RD (comparing to the total number of states 2N) learning process can be extremely slow. It results from the fact that the Q-learning controller needs to reach the goal state for correct learning, but when none of the allowed actions brings the process to the goal state, the algorithm is unable to provide efficient learning.

According to the ε-greedy method, in each iteration of the algorithm, a real value from the range 0-1 is drawn. When that value is smaller than the value eps predefined within the range 0-1, the algorithm switches to the exploration stage, in the opposite situation the exploitation stage is selected and the action with the highest value in the Q-matrix is taken and applied to the process. Thus, by proper adjustment of eps, it is possible to adjust how frequently the Q-learning controller switches between exploration and exploitation. Similarly to the choice of RD, the higher eps is, the faster the learning process is, at the expense of the frequency of applying disturbances (not optimal drawn actions) to the system. Note that for relatively large value of eps, it is possible to successively apply several drawn actions instead of optimal ones, which disturb the process significantly. A complete diagram of the operating Q-learning controller is presented in Fig. 1.

Fig. 1. Flowchart of proposed Q-learning controller algorithm.

# Simulation results

The implementation of the algorithm is verified by simulation for two stable SISO processes          K1s=15s2+2s+1,K2s=15s+12. The first process has a strong oscillatory nature and the second one represents second-order dynamics with two equal time constants. Both processes are relatively difficult to control, especially if the reference trajectory is defined as first-order dynamics. The presented method should be possible to use for every system that can be controlled using a PI algorithm. First. the performance of Q3d using a three-dimensional Q-matrix was compared with the proposed Q2d with a reduced two-dimension Q-matrix for the process K1. The tunings for the existing PI controller were selected as the default settings used for Siemens PLC-based PI controller: KPI = 1, TI = 20. The target trajectory was defined by Te = 8. In both cases, the Q-matrices were initialized using the methods proposed for the respective algorithms to ensure bumpless switching with the existing PI controller. The learning process was divided into learning cycles called epochs, in each epoch the setpoint was randomized and the system was brought to a steady state. The experimental results after 10,000 learning epochs are presented in Fig. 2. In both cases for Q3d and Q2d, the initialization process provides a very accurate representation of the PI controller behavior so the switching between the existing PI controller and each of the considered Q-learning controllers is bumpless. After the learning period, both Q-learning controllers ensure accurate and very similar tracking of the predefined reference trajectory. The modifications proposed for a new Q2d approach do not significantly affect its performance compared to Q3d, despite the significant reduction in the number of states and the Q-matrix dimensionality which was the aim of the new approach. Moreover, there has been an improvement in the learning process because the learning rate increased approximately three times. This fact can be seen in Fig. 3 showing the change in the value of the norm:

norm= i=1nQmatrix i-Qmatrix i-1 ,         (8)

where n is the number of learning epochs.

During the learning process, the values in the Q-matrix converge to certain values and consequently, the norm of the Q-matrix defined by (8) should converge to zero. When the norm values are close to zero, it means that the changes forced in Q-matrix by the learning process are negligibly small and the learning process can be considered complete. For the proposed Q2d approach, rapid decay of the norm value and its relatively fast convergence to the value close to zero can be observed after about 3,000 epochs. After 4 000 epochs, the first negative norm values appear, which can be considered as completing the learning process. On the opposite, for the Q3d approach, the norm value stays close to the value of 0.4 and practically remains unchanged up to 10,000 epochs, showing a constant rate of learning over successive epochs. In terms of dimensionality of the Q-matrix, for the considered example, the size of the Q-matrix was reduced from [191x321x321] for Q3d to only [105x105] for the proposed Q2d approach. In addition, reaching a predefined reference trajectory for the K1 process and the Q3d approach required about 9 000 - 10 000 learning epochs. For proposed Q2d modification, comparable results are obtained after only 2000 – 3000 epochs for both presented processes.

Fig. 2. Comparison of Q3d and Q2d with K1(s) process after 10 000 learning epochs. The upper diagrams show the results for Q3d (process output Y and manipulating variable U). The lower diagram shows the results for the Q2d modification. QwL represents the respective Q-learning algorithm initialized based on PI tunings and QL - performance after learning.

As is shown in Fig. 4, even after only a few hundred epochs, a significant improvement over PI controller performance is evident for the Q2d approach. Namely, an overshoot is reduced as the first result of learning and then, the changes in the process output become closer to the predefined reference trajectory. However, due to the obvious difference in dynamics between predefined first-order reference trajectory and the process dynamics, the difference between reference trajectory and the variations of the process output after learning are still visible. The effects of learning can vary depending on the chosen parameters of the Q-learning controller. The more often actions are drawn, the faster the learning process is, however, surely with a more negative impact on the normal closed-loop operation.

Fig. 3. Comparison of norm change for Q2d and Q3d. The normalized norm was calculated by (8) every 100 learning epochs and approximated to show a trend.

Fig. 4. Improvement in control performance after a different number of learning epochs for the Q2d and K2(s) process. QwL represents Q2d initialized based on PI tunings.

Fig. 5. Trajectory tracking and disturbance rejection for Q2d operating K2(s) process for learning from consecutive load disturbance step changes after 5 000 epochs. QwL represents Q2d initialized based on PI tunings and QL - after learning.

As previously, the Q-matrix was initialized using the tunings of the existing PI controller. Each learning epoch consisted of applying a load disturbance and waiting until the disturbance rejection is completed or a maximal epoch length set as 2000 samples are reached. Compared to learning based only on set-point changes, for similar results, a longer process of learning is required, but still, it is possible to ensure closed-loop dynamics close to the predefined reference trajectory.

Another important factor is the size of the Q-matrix and thus the number of distinguishable states. Increasing the number of states potentially allows for more significant improvement in the control performance. This is also noticeable when initializing the Q-learning algorithm, as a certain number of states are required to ensure bumpless switching using the existing PI controller. In situations requiring higher/lower accuracy, the number of states can be appropriately increased/reduced to meet technological requirements. For better representation of industrial conditions, the ability to learn of Q2d controller based only on load disturbance changes applied to a closed-loop system with amplitude randomized within the range (-0.5, 0.5) was also tested and the results are presented in Fig. 5.

# Conclusions

Based on the general Q-learning framework, this paper presents a new Q2d control algorithm that significantly simplifies the previous Q3d approach. For Q2d, the number of tuning parameters is reduced, which simplifies its implementation by reducing the dimensionality of the Q-matrix, simplifying the initialization process and increasing the learning rate. At the same time, all advantages of the Q3d are preserved with very high accuracy. In detail, the Q2d still does not require any knowledge of the mathematical model of the controlled process and the control goal is defined by the reference trajectory, based on the values of the control error and its time derivative. The proposed initialization of the Q-matrix allows for bumpless switching from the existing PI controller operating the process based only on PI tuning parameters and the sampling time. The results are presented by simulation for the second order system and for the oscillatory system. Modifications proposed for Q2d allow for much more convenient industrial implementation. Simulation results show that for Q2d, the learning process can be based not only on applying intentional setpoint changes but also on the intentional load disturbance changes applied to the closed-loop system. The latter is much more acceptable from a practical viewpoint.

There are still many problems to be solved, for example, the application of the Q-learning method for control of processes with significant time delay. In such cases, after the execution of an action, the Q-learning controller cannot properly reward it due to a lack of simple correlation between the action and its result. Extensive laboratory experiments also should be carried out using the practical implementation in PLC. These issues will be investigated in the future.

##### Acknowledgments

K. Stebel and J. Czeczot were financed by the grant from SUT - subsidy for maintaining and developing the research potential in 2022: J. Musial was financially supported by 02/060/BKM/0037 grant.

##### References

Watkins, C. J. C. H. Learning from delayed rewards, England: University of Cambridge, 1989

Watkins, C. J. C. H., & Dayan, P. Technical note: Q-learning. Machine Learning, 8, 1992, pp. 279–292.

Peng Y., Chen Q., Sun W. Reinforcement Q-Learning Algorithm for H∞ Tracking Control of Unknown Discrete-Time. Linear Systems. IEEE Transactions on Systems, Man, and Cybernetics: Systems, 50, 2020, pp. 4109-4122.

Zamfirache I. A., Precup R-E, Roman R.-C., Petriu E. M. Reinforcement Learning-based control using Q-learning and gravitational search algorithm with experimental validation on a nonlinear servo system. Information Sciences, 583, 2022, pp. 99–120.

Ge Y., Liu X. & Li Y. Optimal control for unknown mean-field discretetime system based on Q-Learning. International Journal of Systems Science, 52, (15), 2021, pp. 3335–3349.

Borkar V. S., Chandak S. Prospect-theoretic Q-learning. Systems & Control Letters, 156, 2021.

Mu C., Zhao Q., Sun C., Gao Z.. An ADDHP-based Q-learning algorithm for optimal tracking control of linear discrete-time systems with unknown dynamics. Applied Soft Computing Journal, 82, 2019.

Chen C., Sun W., Zhao G., and Peng Y. Reinforcement Q-Learning Incorporated With Internal Model Method for Output Feedback Tracking Control of Unknown Linear Systems. IEEE ACCESS, 8, 2020.

Phan, M.Q., Azad, S.M.B. Model predictive control and Model Predictive Q-Learning for structural vibration control. AAS/AIAA Astrodynamics Specialist Conference, 2017.

Phan, M.Q., Azad, S.M.B. (2020). Input-Decoupled Q-Learning for Optimal Control. The Journal of the Astronautical Sciences, 67, 2020, pp. 630–656.

Bettoni D, Soppelsa A, Fedrizzi R. and Toro Matamoros R. M. Analysis and Adaptation of Q-Learning Algorithm to Expert Controls of a Solar Domestic Hot Water System. Appl. Syst. Innov., 2, (15), 2019.

Tiistola S., Ritala R., Vilkko M. Quantized measurements in Q-learning based model-free optimal control. IFAC PapersOnLine, 53-2, 2020, pp.1640–1645.

Zarrabian, S., Belkacemi, R., & Babalola, A. A. Reinforcement learning approach for congestion management and cascading failure prevention with experimental application. Electric Power Systems Research, 141, 2016 pp. 179–190.

Low E. S., Ong P., Cheah K. C. Solving the optimal path planning of a mobile robot using improved Q-learning, Robotics and Autonomous Systems, 115, 2019, pp.143–161.

Kim S. H., Song K. R, Kang Y, Hyon C. On-line set-point optimization for intelligent supervisory control and improvement of Q-learning convergence. Control Engineering Practice 114, 2021.

Stebel K. Practical aspects of the model-free learning control initialization. 20th International Conference on Methods and Models in Automation and Robotics (MMAR), 2015, pp.453-458.

Musiał J., Stebel K., Czeczot J. Self-improving controller for a class of dynamical processes based on Q-learning technique. Archives of Control Sciences, 2021, pp. 527-55.

Stebel K.,  Choinski D. Performance Improvement for Quasi Periodical Disturbances in PH Control. Advances in Electrical and Computer Engineering, 15, (1), 2015, pp. 123-134.

Kozyra, A.,Skrzypczyk, K., Stebel, K., Rolnik, A. Rolnik, P., Kucma, M. Remote controlled water craft for water measurement. MEASUREMENT, 111, 2017, pp. 105-113.

Lam H.-K., Shi Q., Xiao B., and Tsai S.-H. Adaptive PID Controller Based on Q-learning Algorithm. CAAI Transactions on Intelligence Technology, 3(4), 2018, pp. 235–244.

Syafiie S., Tadeo F., and Martinez E. Softmax and "ε-greedy policies applied to process control. IFAC Proceedings, 37, 2014, pp. 729–734.